Cars

Before Gradient:

v0	=33.0;

a	=2.0;

b	=0.9;

T	=1.0;

When Gradient is hit:

v0	=25.0;

a	=0.6;

b	=0.9;

T	=2.0;

Trucks

Before Gradient:

v0	=22.2;

a	=0.6;

b	=0.9;

T	=1.0;

When Gradient is hit:

v0	=30/3.6;

a	=0.1;

b	=2;

| Param | Car |  | Truck |  |
| --- | --- | --- | --- | --- |
|  | Off Gradient | On Gradient | Off Gradient | On Gradient |
| v0 a b T | 33.0 2.0 0.9 1.0 | 25.0 0.6 0.9 2.0 | 22.2 0.6 0.9 1.0 | 8.33 0.1 2.0 2.0 |

T	=2.0;

Steps to take for an uphill gradient (i.e. gradient positive):

- reduce the ability of the vehicles to accelerate by applying an external deceleration of -gradient/10 to accel function;

- increase the safe time headway to reflect drivers’ awareness of their reduced efficacy;

- reduce the desired speed to reflect the reduced maximum velocity of the vehicles – but how to implement if in speed limit section?

- Increase the ability of the vehicle to brake by increasing b by gradient/10

In one paper Treiber reduces v0 from 120 to 60 km/h for an uphill gradient.

Solution:

Step 1

Change the ability of a vehicle to accelerate by applying – gradient/10 to m_A as a resistance acceleration in Vehicle.

Step 2

Following Treiber increase T from 1 to 2.0 s for, say 0 to 10% gradient, so in general:

T = T + gradient/T_GRADIENT_SLOPE secs

Where T_GRADIENT_SLOPE = 10 but can be changed as needed.

Step 3

Similar to T, reduce velocity by say, 50 km/h for a 10% gradient, so in general:

V0 = V0 – gradient / (V0_GRADIENT_SLOPE * 3.6) // change to m/s

Where V0_GRADIENT_SLOPE = 10 / 50 = 0.2 but can be changed as needed.

Step 4

Similarly to Step 1 – but add the resistance acceleration to the braking ability, b.

Lastly – introduce user flags: A_GRADIENT_FLAG and B_GRADIENT_FLAG = true or false if the gradient should affect IDM parameters a or b.